HyperLapse Cart — Exposure Cycle Redesign

Meter → PUT → Fire, gated on addedcontents

Design note for build. Canon EOS R3 / CCAPI v1.4.0. Drafted 06 Jun 2026.

# 1. Problem

The live cadence fires the shutter, then issues the luminance fetch and the Tv/ISO PUT immediately after (fetch_delay_ms = 0). Those post-fire calls land inside the camera's busy window — the period after a press when the camera is still completing the exposure and pipeline settle. Write/control CCAPI in that window is rejected with HTTP 503 "During shooting or recording", which then triggers the Tv/ISO PUT retry loop (5x, 3 s initial backoff) — a cadence-killer.

The fix is not to fight the busy window with a delay, but to stop issuing CCAPI into it at all.

# 2. What was measured

Busy window measured directly via /debug/busywindow: fire one shot, then probe until the camera stops returning 503. The window scales as exposure time plus ~1 s of pipeline settle (Long-Exposure NR off, confirmed — no dark-frame doubling).

| Tv (exposure) | Measured busy window | Cadence interval | Idle gap |
| --- | --- | --- | --- |
| 0.5 s | ~1.14 s (1140-1152 ms, 3 runs) | 2 s | ~0.85 s |
| 20 s | 21.19 s (21187 ms) | 22 s | ~0.8 s |

Critically, the gating is by call class, not uniform:

| Call class | Gated during shot? | Evidence |
| --- | --- | --- |
| GET (settings/tv read) | No - returns 200 immediately | probe at ~210-480 ms, 0x 503 |
| PUT (Tv/ISO write) | Yes - 503 until window clears | 503 'During shooting or recording' |
| POST (next fire) | Yes - rejected mid-capture | ref 4.8.1 'process cannot be accepted' |

So a luminance GET is never blocked, but the Tv/ISO PUT and the next fire are. The ~1 s settle is not card-write time (the R3 sustains 30 fps to a fast 2 TB CFexpress, so a single RAW writes in well under 100 ms) — it is CCAPI control-acceptance settle.

# 3. How often change is actually needed

From the Excel fallback schedule (FallbackFormula sheet), the tightest Tv/ISO crossover spacing anywhere in dusk or dawn is 60 seconds — one change per minute at the very busiest. Tv ceiling 20 s, ISO ceiling 1600, ISO base 100.

- Light changes slowly: at most one Tv-or-ISO step per minute.

- Metering every ~20 s is 3x oversampling — enough to catch a 60 s-spaced crossing promptly.

- Most metering reads (about 2 of 3) result in no change. The waste is read frequency, not per-read cost.

The luminance read is the heaviest CCAPI call (~200 ms, ~2.6 KB liveview histogram) and the most frequent. It already replaced a multi-second download-JPG-to-Python path, so it is not slow in absolute terms — it is just run more often than needed.

# 4. The design

## 4.1 Reverse the cycle order

Move all camera control to the TOP of the cycle, when the camera is guaranteed idle (a full interval has passed since the last shot):

- Meter (if a metering tick is due): luminance GET.

- Adjust (if the read crosses a threshold): a single Tv-or-ISO PUT. Never both in one cycle.

- Fire: the shutterbutton POST.

This is the same ordering soak mode already uses and documents. Because nothing talks to the camera after the fire, the busy window is never hit — no 503, no retry storm, no Tv-formula delay, no readiness polling. It is also more correct: the frame is shot with the freshly-adjusted settings, instead of adjusting a frame late.

## 4.2 Gate the next cycle on addedcontents (not a delay)

The camera reports shot completion directly. CCAPI Reference §6.2.27 (Event handling → Add contents): after a shutterbutton POST, GET /event/polling returns addedcontents listing the new file once it is written to card. This is the exact, format-agnostic "shot done" signal.

- event/polling is a GET, so it is not gated during the shot (measured).

- Use ?timeout=immediately so the poll returns at once rather than long-polling.

- It self-scales from 1/5000 s to 20 s automatically — no Tv-derived constant, no measured margin.

- addedcontents path form differs by version (ver1.0 = full URL, ver1.1+ = path); the R3 returns the path form.

So between the fire and the next cycle's top-of-cycle CCAPI, poll event/polling until addedcontents appears. That confirms the camera is idle again, replacing the busy-window guess entirely.

## 4.3 Metering schedule

Decouple metering from photo count. Meter on a wall-clock interval (~20-30 s), expressed so it self-scales across regimes:

- Dusk (~5 s cadence): meter every ~20 s = roughly every 4th frame. Plenty of idle slots.

- Near night (Tv up to 20 s, ~22 s interval): a 20 s metering interval lands near once per frame, which is fine — changes are still only ~once/minute, and near-night frames are sparse (~3/min).

- During a long exposure there is no valid live image, so the luminance read must sit in the inter-frame gap. With the reversed order it already does (top of cycle, camera idle).

# 5. Near-night squeeze and the one lever

At Tv = 20 s the interval is 22 s and the busy window is 21.19 s, leaving only ~0.8 s idle. A luminance read plus PUT (~0.5 s) fits, but with little margin. The single sizing lever is the interval buffer in intervalForTv():

- Current: interval = max(2 s, ceil(Tv + 1.5)). At Tv = 20 that is 22 s → ~0.8 s idle.

- Proposed: raise the buffer at long Tv (e.g. +2.5 to +3 s) → ~2-3 s idle → comfortable room for the top-of-cycle meter + PUT.

This is one constant, and it only matters at the long-Tv end. With addedcontents gating, even a tight gap is safe (the poll proves readiness), so the buffer bump is a comfort margin, not a correctness requirement.

# 6. Build plan

- Reverse the live cadence service order to meter → PUT → fire (mirror soakCycle).

- Replace the post-fire fetch/PUT timing (fetch_delay_ms, the Tv+margin idea) with an event/polling addedcontents gate after the fire.

- Change metering from every-Nth-photo to a wall-clock interval (default ~20-30 s).

- Optionally raise the intervalForTv buffer at long Tv (+1.5 → +2.5/+3 s) for near-night idle margin.

- Retire the /debug/busywindow PUT-probe scaffolding and the busy-window measurement code once the above lands.

# 7. Confirmed decisions (no longer open)

- Luminance from the liveview histogram is independent of capture format. Histogram is liveviewdata.histogram (YRGB x 256) from the liveview stream; stillimagequality raw/jpeg are separate fields. Dropping dual (RAW+JPEG) capture did not remove the histogram — it was the correct call and also shortens the per-shot write. (CCAPI ref §5.2, §6.2.16.)

- 400 = 'Invalid parameter' (illegal af value). Our {"af":false} is valid; the earlier wired 400s were a segmented request line, fixed by the single-buffer write.

- 503 family = Device busy / During shooting or recording / Mode not supported / Taken in preparation. (CCAPI ref §4.8.1.)

# 8. One item to verify in real light

Bench runs showed luminance mean = 0, almost certainly the dark/capped bench rather than a RAW-only artifact. Confirm the RAW-only liveview histogram reads true under real light before relying on it for the exposure walk.